CANopen

| iDTRONIC GmbH Ludwig-Reichling-Straße 4 67059 Ludwigshafen Germany/Deutschland Phone: +49 621 6690094-0 Fax: +49 621 6690094-9 E-Mail: info@idtronic.de Web: idtronic.de | Issue 0.2 – 21. July 2026 – Subject to alteration without prior notice. © Copyright iDTRONIC GmbH 2026 Printed in Germany |
| --- | --- |

Contents

1	Function Description	4

1.1	RFID Function	4

1.2	LED Function	4

1.3	LED Control	4

1.4	Configure Bus Address	4

2	Hardware	5

2.1	Pinout	5

2.2	Connector Orientation	5

# Function Description

## RFID Function

Automatically read UID of ISO14443A and ISO15693 type tags. Send the UID with a size of 4, 7 or 8 Bytes in a single telegram. Repeat this telegram as long as a tag can be detected.

## LED Function

After power on, all LEDs shall be OFF.

## LED Control

Command:	Byte 0: 	Length of UDS data

Byte 1:	 	UDS Service (0x2E WriteDataByIdentifier)

Bytes 2-3:	DID (e.g. 0x6033)

Byte 4: 	LED Control bits:	0x00 = All LEDs OFF

0x01 = only red

0x02 = only green

0x03 = red and green

So, the coding for the LEDs is in the 2 lowest bits.

Example

>> 04 2E 60 33 02 – light only the green LED

## Configure Bus Address

Factory preset address is 0x36.

The address 0x00 is recognized as universal address, so the reader reacts always to it.

Command:	Byte 1: 	Length of UDS data (0x05)

Byte 2: 	UDS Service (0x2E WriteDataByIdentifier)

Bytes 2-3:	DID (0x6091)

Bytes 4-5:	Device Address (e.g. 0x0037 = Address 37)

Example

>> 05 2E 60 91 00 37 – sets the device address to 0x37

# Hardware

## Pinout

1 = Shield, PE

2 = Power Supply +

3 = GND for Power Supply and CAN

4 = CAN High

5 = CAN Low

250 kbs

## Connector Orientation

The connector shall be rotated, so that an angled connector will have the cable outlet in direction of the bottom of the housing. Please see the following pictures: